Table 1.

Environmental impact scenario

| Environmental impact scenario | Characteristics of impact | Value of impact |
| --- | --- | --- |
| Normal | Constant work in “continuous mode” of all technological installations, technical infrastructure. Main process and supporting ones are executed without disruption in scheduled mode. | 1 |
| Emergency | Emergency situation of emergency nature, not planned, which threatens the safety of people, goods/properties and environment. | 2 |
| Special | Incidental and planned replacement or maintenance activities required to ensure continuity and security of the organization and processes. | Points are not awarded |

Source: Own elaboration based on company’s documentation

Appendix 1.

EFMEA worksheet

| Process/Area | Subprocess/ Activity / Operation | Aspect | Quantity per annum (2020) | Quantity per annum (2021) | Change Y2Y | Impact on the environment | Methods of supervision over aspects / Operational control | Monitoring methods | NS | ES | SS | Key Legal or other requirements | OCC | SEV | IMP | RPN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| IM | Maintenance of emergency power generator | Storage of backup supply of diesel oil | Consumption when needed (in litters) | Consumption when needed (in litters) | +17% | Pollution of the environment, soil and water contamination | Maintenance activities, ongoing supervision | Verification during the service |  | X |  | Act 7 | 1 | 1 | 1 | 2 |
| IM | ICT maintenance | Replacement of computer parts, computer batteries and UPS | negligible quantities - relatively new equipment | negligible quantities - relatively new equipment | - | Pollution of the environment | Equipment servicing by a specialized company | Equipment servicing by a specialized company |  | X |  | Act 1 | 1 | 1 | 2 | 4 |
| IM | Infrastructure servicing | Waste generation during periodic repairs and maintenance of machines and devices | Aspect under the supervision of service contractors (minimal registered own consumption per year | Aspect under the supervision of service contractors (minimal registered own consumption per year | - | Pollution of the environment | Supervision over service providers, waste generation monitoring | Service providers’ declaration reg. waste management. Waste log book based on EWC codes |  |  | X | Act 1 | 2 | 2 | 2 | 8 |
| IM | Service of forklifts, scrubbers, sweepers, wrappers | Battery wear | Based on individual wear of the battery | None | Minor changes | Pollution of the environment | Regular maintenance checks and disposal at the end of service life - used batteries are returned the provider. | Monitoring the level of wear and damage |  |  | X | Act 2 | 1 | 2 | 1 | 2 |
| IM | Cooling system maintenance | Refrigeration and air conditioning devices - the use of refrigerants (freons) - ozone-depleting substances. | The refrigerant is replenished in the event of a failure | Freon 410 A | Minor changes | Ozone-depleting substances | Regular inspections (twice a year) and leak testing, installation labelling, maintenance activities carried out by authorized suppliers | Tightness control, register of the replenishments |  | X |  | Act 3 Act 4 | 1 | 3 | 3 | 18 |
| IM | Infrastructure servicing | oils, greases, used spare parts | Under the supervision of service providers | Under the supervision of service providers | - | Pollution of the environment | Supervision over service providers | Service providers’ declaration reg. waste management. | X |  |  | Act 1 | 2 | 2 | 1 | 4 |
| IM | Cargo handling | Hydrogen emission during battery charging. | - | - | - | Pollution of the environment, threats for human wellbeing, as well as for the safety of the infrastructure. | Maintenance activities, permanent supervision over the devices. | Charging room is equipped with hydrogen sensors, emergency ventilation system, emergency power cut-off system. |  | X |  | Regulation 1 Regulation 2 | 1 | 3 | 3 | 18 |
| IM | Cargo handling | Battery acid leaks | - | - | No change | Pollution of the environment, threats for human wellbeing, as well as for the safety of the infrastructure | Maintenance activities, permanent supervision over the devices. | Charging room is equipped with drainless tanks, environmental first aid kit |  | X |  | Regulation 1 | 1 | 3 | 1 | 6 |
| IM | Facility lightning | Used fluorescent lamps | In kilograms | In kilograms | No changes | Pollution of the environment. | Selective waste collection, storage in designated, labelled containers. Return of used ones to the point of sale. | One to one replacement of used one by new ones. | X |  |  | Act 1 Act 5 | 1 | 1 | 1 | 1 |
| IM | Rainwater and snowmelt drainage to the sewage system | Sewage system | Aspect under control of the landlord | Aspect under control of the landlord | - | Soil and water pollution | Regular inspections of sewerage system, and sewage separators. | Supervision over service providers | X |  |  | Regulation 1 | 3 | 1 | 3 | 9 |
| IM | Pest management | DDD | According to the GHP/GMP pest procedures (P1ZS) | According to the GHP/GMP pest procedures (P1ZS) | - | Negative impact on free-living animals. | Traps are serviced by the service provider | According to the GHP/GMP pest procedures (P1ZS) | X |  |  | Act 6 | 3 | 1 | 1 | 3 |
| IM | Floor maintenance and conservancy | Chemicals usage | According to the GHP/GMP pest procedures (P1MD/I1MD) | According to the GHP/GMP pest procedures (P1MD/I1MD) | - | Soil and water pollution | Storage and usage of the chemicals according to the safety data sheets. | Registers of chemicals usage. Monitoring of the sewage water parameters. | X |  |  | Act 7 | 3 | 1 | 1 | 3 |
| IM | Compressors maintenance | Oil | In litters | In litters | No changes | Soil and water pollution | Supervision over service providers | Used oil verification done by service provider. Service providers’ declaration reg. waste management. |  |  | X | Act 7 Regulation 3 | 1 | 2 | 1 | 2 |
| BA | Employees’ activity | Generation of social and household sewage | In cubic meters | In cubic meters | Minor changes | Pollution of the environment. | Sewage system | Monitoring of usage | X |  |  | Act 7 Act 8 | 3 | 2 | 3 | 18 |
| BA | Cargo storage in temperature controlled conditions. | Electric energy usage | In MWh | In MWh | Minor changes | Depletion of natural resources. Pollution of the environment. | Regular checks and proper maintenance. Optimalization of usage. | Monitoring of usage. | X |  |  | Act 7 | 3 | 2 | 3 | 18 |
| BA | Cargo storage in temperature controlled conditions. | Water supply usage. | In cubic meters | In cubic meters | Minor changes | Depletion of natural resources | Regular checks and proper maintenance. Optimalization of usage. Parameters monitoring. Separation of water and oil. | Monitoring of usage. | X |  |  | Act 6 Act 7 Act 8 | 3 | 2 | 3 | 18 |
| BA | Cargo storage in temperature controlled conditions. | Operational work of devices and installations. | In dB | In dB | No changes | Noise emission to the environment | Regular checks. Emission monitoring, | Regular noise emission measurements | X |  |  | Regulation 2 Act 7 | 3 | 1 | 1 | 3 |
| BA | Cargo storage in temperature controlled conditions. | Reirrigation of the cargo – leakage of the ammonia system. | - | - | - | Pollution of the environment. Threats for human life and wellbeing, as well as for the safety of the infrastructure. Threat for the stored frozen food cargo. | Permanent monitoring. Regular maintenance | Ongoing monitoring of the operation of cooling devices. Automated notification system in case of irregularities or malfunctions of the ammonia system. Drainless tank for the ammonia. Dry riser to neutralize the ammonia cloud. Ammonia detection sensors system. |  | X |  | Regulation 1 | 1 | 3 | 3 | 18 |
| BA | Cargo storage in temperature controlled conditions. | Forklift LPG cylinder leakage. | - | - |  | Pollution of the environment. Threats for human life and wellbeing, as well as for the safety of the infrastructure. | Regular maintenance | Storage in the designated safe zone. Operational checks before use. |  | X |  | Regulation 1 | 1 | 2 | 2 | 8 |
| BA | Cargo storage in temperature controlled conditions. | Utilization of not fit to human consumption cargo | In kilograms | In kilograms | Minor changes | Threat for the food security. | Permanent monitoring and reporting according to the GMP procedures (P1MP). | Weekly reporting according to the GMP procedures (P1MP). Verification of the cargo during intake according to GMP procedures (P1ZZ). |  |  | X | Act 6 | 1 | 1 | 2 | 2 |
| BA | Cargo handling | Waste generation | In Mg | In Mg | Minor changes | Pollution of the environment. | Waste management based on procedures (P1UO, P2UO) | Regular reporting. | X |  |  | Act 1 Act 7 Act 9 | 3 | 2 | 2 | 12 |
| BA | Cargo handling | Propane butane gas for forklifts | In Mg | In Mg | Minor changes | Pollution of the environment. | Regular maintenance | Storage in the designated safe zone. Operational checks before use. | X |  |  | Act 1 | 1 | 2 | 2 | 4 |
| Business activity | Facility heating | Electric energy usage | In MWh | In MWh | Minor changes | Depletion of natural resources. Pollution of the environment. | Regular checks and proper maintenance. Optimalization of usage. Recuperation of waste heat from refrigeration system. | Monitoring of usage. Regular maintenance checks | X |  |  | Act 7 | 3 | 1 | 3 | 9 |
| Business activity | Company’s car usage | Consumption of gasoline and diesel fuel. | In Mg | In Mg | Minor changes | Depletion of natural resources. Pollution of the environment. | Regular service checks. | Regular monitoring and reporting. | X |  |  | Act 7 | 3 | 2 | 2 | 12 |
| Business activity | Documentation pritining | Paper consumption | Reams of paper | Reams of paper | No changes | Depletion of natural resources. | Waste segregation. Recycling of printers’ toners. | Regular monitoring. | X |  |  | Act 1 | 1 | 1 | 1 | 1 |

Legend:

IM- Infrastructure maintenance; BA – Business activity

NS – Normal Scenario; ES – Emergency Scenario; SS – Special Scenario; OCC – Occurrency; SEV – Severity; IMP - Impact

Act 1 - Ustawa z dnia 14 grudnia 2012 r. o odpadach (Dz. U. 2021 poz. 779); Act 2 - Ustawa z dnia 21 grudnia 2000 r. o dozorze technicznym (Dz. U. 2021 poz. 779); Act 3 - Ustawa z dnia 15 maja 2015 r. o substancjach zubożających warstwę ozonową oraz o niektórych fluorowanych gazach cieplarnianych (t.j. Dz.U. 2020 poz. 2065); Act 4 - Ustawa z dnia 12 lipca 2017 r. o zmianie ustawy o substancjach zubożających warstwę ozonową oraz o niektórych fluorowanych gazach cieplarnianych oraz niektórych innych ustaw (Dz.U. 2017 poz. 1567); Act 5 - Ustawa z dnia 11 września 2015 r. o zużytym sprzęcie elektrycznym i elektronicznym (Dz.U. 2020 poz. 1893); Act 6 - Ustawa z dnia 25 sierpnia 2006 r. o bezpieczeństwie żywności i żywienia (Dz.U. 2020 poz. 2021); Act 7 - Ustawa z dnia 27 kwietnia 2001 r. Prawo ochrony środowiska (Dz.U. 2021 poz. 1973); Act 8 - Ustawa z dnia 20 lipca 2017 r. - Prawo wodne (Dz.U. 2021 poz. 2233); Act 9 - Ustawa z dnia 13 września 1996 r. o utrzymaniu czystości i porządku w gminach (Dz.U. 2021 poz. 888); Regulation 1 - Rozporządzenie Ministra Infrastruktury z dnia 12 kwietnia 2002 r. w sprawie warunków technicznych, jakim powinny odpowiadać budynki i ich usytuowanie (Dz. U. 2022 poz. 1225); Regulation 2 - Rozporządzenie Ministra Zdrowia z dnia 11 października 2019 r. zmieniające rozporządzenie w sprawie badań i pomiarów czynników szkodliwych dla zdrowia w środowisku pracy (Dz.U. 2019 poz. 1995); Regulation 3 - Rozporządzenie Ministra Gospodarki z dnia 5 października 2015 r. w sprawie szczegółowego sposobu postępowania z olejami odpadowymi (Dz.U. 2015 poz.1694).

Source: Own elaboration based on company’s documentation